AEROLAB

Faculty Demonstration & Graph Walkthrough

# The one-sentence takeaway

AeroLab connects ESP32 control, sensors, a dashboard, and software fault simulations. Today’s graphs support model/simulation claims; independent hardware validation is the next research step.

# Suggested 5-minute faculty walkthrough

| Time | Show / explain | Suggested wording |
| --- | --- | --- |
| 0:00–0:40 | Dashboard source: DEMO, simulation, or live. | I’ll distinguish simulation from physical telemetry. |
| 0:40–1:25 | MATLAB → ESP32 → sensors/actuator → dashboard. | The contribution is an integrated, auditable teaching demonstrator. |
| 1:25–2:10 | US1 fixed, US2 servo-mounted; echo validity and raw LDR. | This is angular-sector proximity response, not continuous hand pose. |
| 2:10–3:15 | Step plot: linear models versus clipped response. | This is a model benchmark, not measured servo performance. |
| 3:15–4:05 | Observer/residual/CUSUM with injected-fault marker. | The offset is injected in software; residual is not independent sensor validation. |
| 4:05–5:00 | State the next validation experiment. | I’ll measure servo angle independently and report repeatable timing/detector statistics. |

# Terminology guardrails

| Say | Avoid until measured |
| --- | --- |
| Model-linked demonstrator | Validated digital twin / flight controller |
| Servo ultrasonic sector scan | Continuous 2-D hand tracking |
| Software-injected drift | Hardware fault-detection validation |
| Raw, uncalibrated LDR ADC | Lux / calibrated photometry |
| Nominal 20 Hz loop | Verified end-to-end timing |
| Simulated parameter sweep | Guaranteed hardware robustness |

# How to read each graph

| Graph / scope | What to point out | What it means / caveat |
| --- | --- | --- |
| Step-response comparison | Setpoint and continuous, discrete-linear, and saturated curves. | Compare rise, overshoot, settling, and input bounds. Saturated trace is closer to firmware command logic, but remains model-only; do not quote it as measured servo dynamics. |
| Twin/model-state coincidence | Overlapping x₁ and twin-state traces; divergence when software offset is introduced. | Internal states share model propagation/control. Agreement is not independent validation of a physical plant. |
| Residual and CUSUM | Residual magnitude against δ; positive/negative accumulators against h; injection time. | Illustrates detector logic under a chosen synthetic fault/noise. It does not establish hardware false-alarm probability or detection latency. |
| Control effort and servo | Normalized u in [0,1] and servo-angle command. | Shows commanded effort/angle mapping. Distinguish command from independently measured shaft position. |
| Observer estimation error | Position error and scaled velocity error decay in simulated run. | Model observer behavior only; no physical reference measurement is present. |
| Bode / root locus | Frequency response and nominal model pole movement under plotted assumptions. | Useful design analysis for the assumed model; not an empirical stability margin or robustness certificate for the hardware. |
| Parameter sweep | Spread of model responses under the stated parameter perturbations. | Sensitivity study on simulated plants. Do not state as measured reliability or a guarantee. |
| Attitude / sensor scopes | Roll/pitch, vibration, distance, temperature/gas, raw LDR. | Live only when source and freshness indicate hardware. LDR is ADC counts until calibrated; ultrasonic channels require fresh valid echoes. |

# Dashboard cleanup before the meeting

Keep the source indicator visible; never present DEMO/simulation as LIVE.

Use corrected analytics: assumed model parameters, firmware-matched gains, and no unsupported FAR/robustness claims.

Call MATLAB views model analysis. Label residual/CUSUM as software-model injected simulation.

For hardware demo, verify fresh packet age and sensor validity; record CSV. Connection alone is not validation.

Describe hand mode as sector scan/lock; verify US2 echo, servo motion, reacquisition, alarms, and limits.

Call LDR/MQ raw or uncalibrated unless calibration and warm-up are documented.

# Likely faculty questions

| Question | Short answer |
| --- | --- |
| Is it a digital twin? | Not yet validated: residual uses internal model states plus injected drift, not an independent actuator measurement. |
| Does it track a hand? | It scans and locks/reacquires a range sector; one sensor cannot recover continuous 2-D hand pose. |
| Where did ωₙ, ζ come from? | They are design assumptions; identify them from measured servo/load data. |
| What does the alarm prove? | Software response to coded synthetic offset only; hardware rates remain unknown. |
| Strongest result? | Integrated model-to-firmware-to-dashboard teaching demonstrator. |
| Next experiment? | Measure shaft angle independently, synchronize logs, identify/validate model, test held-out faults. |

# A strong next validation package

| Validation area | Measure before a performance claim |
| --- | --- |
| Hardware provenance | Board and firmware version, supply voltage, wiring, sensor identity, environment, calibration procedure, and test date. |
| Independent actuator ground truth | Encoder or optical angle reference synchronized with command and telemetry timestamps. |
| Model identification | Repeated step/frequency tests; fit residuals and validation on separate runs, including deadband, rate limits, and saturation. |
| Runtime timing | Packet age, missed packets, loop-period/jitter distribution, command-to-measurement delay, and reset/error logs. |
| Fault detector | Baseline/fault scenarios; false alarms per hour, detection probability vs fault magnitude, delay distribution, misses, sample count, and confidence intervals. |
| Photonic sensor | LDR ADC-to-lux calibration against a reference meter at multiple levels; document geometry and ambient-light control. |
| Hand interaction | Define target distance/angle/speed and score sector detection/reacquisition. Continuous 2-D tracking needs adequate additional sensing geometry. |